protocolv1.0 - English

(TW66GW02)  Mathis Wireless Protocol Document

1. Basic Information

| Item | Description |
| --- | --- |
| Interface | The wireless module uses USART1/COM1 at 115200 bps, 8 data bits, 1 stop bit, no parity, and no hardware flow control. RX/TX use DMA. |
| Frame Format | Byte[0]=0xFE header, Byte[1]=command, Byte[2..n-2]=payload, Byte[n-1]=0xFF tail. No checksum is implemented. |
| RX Filtering | WirelessTask only accepts frames with length 4..1280 and valid header/tail bytes. Invalid frames are discarded. |
| Scheduling Note | In main.c, WirelessTask is currently commented out: // SCH_Add_Task(WirelessTask, 0, 20); if it remains disabled, received wireless frames will not be parsed periodically. |
| Temperature Encoding | Protocol temperatures are usually split into three decimal digits: hundreds, tens, ones. Example: 175 is encoded as 0x01,0x07,0x05. |
| Special Temperature | HaveTempErr=999, TempHigh=998, TempLow=997, and TempDisconnected=-30 are treated as special values. Most wireless replies report them as 960, encoded as 0x09,0x06,0x00. |
| Unit Definition | Internal units use unitF=0 and unitC=1. Wireless protocol values use 0x01=Fahrenheit and 0x02=Celsius. |
| Setpoint Range | WL_TEMP_MIN=175 and WL_TEMP_MAX=455. Increment, decrement, and direct set operations clamp the setpoint to 175..455. |

2. Function - Byte/Bit - Value - Effect

| Function | Byte / Bit | Value | Effect |
| --- | --- | --- | --- |
| Wireless_Init | COM1.rxFlag | 0 | Clears the wireless UART receive-complete flag. |
|  | COM1.rxLen | 0 | Clears the wireless receive length. |
|  | s_bt_status | 0 | Clears the wireless module cached Bluetooth status. |
| WirelessTask | COM1.rxFlag | 0 | No pending wireless frame; returns immediately. |
|  | COM1.rxFlag | 1 | Reads COM1.rxLen and COM1.rxBuf, clears the flag, validates header/tail, then dispatches the frame. |
|  | RX len | <4 or >1280 | Invalid length; the whole frame is discarded. |
|  | RX Byte[0] | 0xFE | Valid frame header; any other value discards the frame. |
|  | RX Byte[len-1] | 0xFF | Valid frame tail; any other value discards the frame. |
| wl_send | TX channel | USART1_SendData | Sends the wireless reply frame through USART1 DMA. |
| wl_ack_cmd2 | TX Byte[0] | 0xFE | Acknowledgement frame header. |
|  | TX Byte[1] | cmd | Echoes the acknowledged command byte. |
|  | TX Byte[2] | p2 | Echoes the second command payload byte, usually rx[2]. |
|  | TX Byte[3] | 0xFF | Acknowledgement frame tail. |
| wl_dispatch | RX Byte[1] command | 0x01 | Calls wl_handle_power to process power on/off. |
|  | RX Byte[1] command | 0x03 | Calls wl_handle_temp_up to increase the temperature setpoint. |
|  | RX Byte[1] command | 0x04 | Calls wl_handle_temp_down to decrease the temperature setpoint. |
|  | RX Byte[1] command | 0x05 | Calls wl_handle_set_temp to directly set the temperature setpoint. |
|  | RX Byte[1] command | 0x06 | Calls wl_handle_jump_set and returns the current setpoint. |
|  | RX Byte[1] command | 0x07 | Calls wl_handle_query_probe to query probe temperature. |
|  | RX Byte[1] command | 0x09 | Calls wl_handle_unit to switch temperature units. |
|  | RX Byte[1] command | 0x0B | Calls wl_handle_query_status and sends three status frames: 0x1B, 0x2B, and 0x3B. |
|  | RX Byte[1] command | 0x0D | Calls wl_handle_query_set_all to query all set temperatures. |
|  | RX Byte[1] command | 0x0E | Calls wl_handle_query_act_all to query all actual temperatures. |
|  | RX Byte[1] command | 0x24 | Updates Bluetooth connection state and sends no acknowledgement. |
|  | RX Byte[1] command | 0x5F | Calls wl_handle_fw_info to query firmware/capability information. |
|  | RX Byte[1] command | Other | Default path calls wl_ack_cmd2(rx[1], rx[2]) and echoes the command and rx[2]. |
| wl_get_probe_temp | g_probe_connected | 0 | Probe is considered disconnected; returns HaveTempErr=999. |
|  | g_probe_connected | Non-zero | Returns system_data.pt1000_temp[3] as probe temperature. |
| wl_is_special_temp | Input temperature t | 999 | HaveTempErr; treated as a special temperature. |
|  | Input temperature t | 998 | TempHigh; treated as a special temperature. |
|  | Input temperature t | 997 | TempLow; treated as a special temperature. |
|  | Input temperature t | -30 | TempDisconnected; treated as a special temperature. |
|  | Input temperature t | Other | Normal temperature; reported as three decimal digits. |
| wl_get_temp_by_channel | ch/rx[2] | 0x01 | Returns probe1, i.e. wl_get_probe_temp(). |
|  | ch/rx[2] | 0x02 | probe2 is unsupported; returns HaveTempErr=999. |
|  | ch/rx[2] | Other | Unknown channel; returns HaveTempErr=999. |
| wl_get_display_main_temp | DisplayMode | DISPLAY_MODE_D_SURFACE(0) | Returns system_data.pt1000_temp[0], D-surface temperature. |
|  | DisplayMode | DISPLAY_MODE_O_SURFACE(1) | Returns system_data.pt1000_temp[1], O-surface temperature. |
|  | DisplayMode | DISPLAY_MODE_PROBE(2) | Returns probe temperature. |
|  | DisplayMode | DISPLAY_MODE_CAVITY(3) | Returns system_data.pt1000_temp[2], cavity temperature. |
| wl_split3 | Input v | >999 | Clamps v to 999, then splits it into hundreds, tens, and ones. |
|  | Output h/t/l | 0..9 | h=v/100, t=(v/10)%10, l=v%10. |
| wl_handle_power | RX Byte[1] | 0x01 | Power command. |
|  | RX Byte[2] | 0x01 | Power on: if work_process==idle, changes to start_up; replies FE 01 01 FF. |
|  | RX Byte[2] | 0x02 | Power off: if work_process!=idle, changes to shutdown; replies FE 01 02 FF. |
|  | RX Byte[2] | Other | Does not change work state, but still notifies local interaction and echoes the value. |
| wl_handle_temp_up | RX Byte[1] | 0x03 | Temperature setpoint increment command. |
|  | RX len | <7 | Frame too short; returns without acknowledgement. |
|  | RX Byte[2] | 0x01 | Valid target; InTempSet can be increased. |
|  | RX Byte[2] | Other | Invalid target; returns without acknowledgement. |
|  | RX Byte[3] | 0..255 | Increment step; InTempSet += step, then clamps to 175..455. |
|  | TX Byte[3..5] | h,t,l | Returns the updated and clamped setpoint as three digits. |
| wl_handle_temp_down | RX Byte[1] | 0x04 | Temperature setpoint decrement command. |
|  | RX len | <7 | Frame too short; returns without acknowledgement. |
|  | RX Byte[2] | 0x01 | Valid target; InTempSet can be decreased. |
|  | RX Byte[2] | Other | Invalid target; returns without acknowledgement. |
|  | RX Byte[3] | 0..255 | Decrement step; subtracts if possible, otherwise sets 175; then clamps to 175..455. |
|  | TX Byte[3..5] | h,t,l | Returns the updated and clamped setpoint as three digits. |
| wl_handle_set_temp | RX Byte[1] | 0x05 | Direct temperature setpoint command. |
|  | RX len | <7 | Frame too short; returns without acknowledgement. |
|  | RX Byte[2] | 0x01 | Valid target; Byte[3..5] are used to calculate the new setpoint. |
|  | RX Byte[2] | Other | Does not modify the setpoint, but still echoes the original payload. |
|  | RX Byte[3..5] | h,t,l | Calculates v=h*100+t*10+l; clamps to 175..455 and writes InTempSet. |
|  | TX Byte[2..5] | Original rx[2..5] | Echoes target and digits exactly as received; note this is not the clamped InTempSet. |
| wl_handle_jump_set | RX Byte[1] | 0x06 | Jump-to-set / enter-set command. |
|  | RX Byte[2] | Any | Not validated by code; echoed in the response. |
|  | TX Byte[3..5] | h,t,l | Returns current InTempSet as three digits. |
| wl_handle_query_probe | RX Byte[1] | 0x07 | Probe temperature query command. |
|  | RX Byte[2] | 0x01 | Queries probe1; returns pt1000_temp[3] when connected and normal. |
|  | RX Byte[2] | 0x02 | probe2 is unsupported; returns special temperature 960. |
|  | RX Byte[2] | Other | Unknown probe channel; returns special temperature 960. |
|  | TX Byte[3..5] | 0x00,0x00,0x00 | Reserved fields, fixed to 0. |
|  | TX Byte[6..8] | h,t,l or 0x09,0x06,0x00 | Normal temperature is reported as digits; error/disconnect/high/low/unsupported are reported as 960. |
| wl_handle_unit | RX Byte[1] | 0x09 | Temperature unit switch command. |
|  | RX Byte[2] | 0x01 | Switches to Fahrenheit: system_data.units=unitF. |
|  | RX Byte[2] | 0x02 | Switches to Celsius: system_data.units=unitC. |
|  | RX Byte[2] | Other | Does not change the unit, but still notifies local interaction and echoes the value. |
| wl_handle_query_status | RX Byte[1] | 0x0B | Status query command. |
|  | RX Byte[2] | 0x01 | Valid query; sends three status frames: 0x1B, 0x2B, 0x3B. |
|  | RX Byte[2] | Other | Returns without acknowledgement. |
| wl_handle_query_status / 0x1B frame | TX Byte[2] | 0x01 | work_process==set; device is in set mode. |
|  | TX Byte[2] | 0x02 | Current work_process is not set. |
|  | TX Byte[3] | 0x01 | Device is neither idle nor shutdown; treated as powered/running. |
|  | TX Byte[3] | 0x02 | work_process==idle or shutdown; treated as off/standby. |
|  | TX Byte[4..11] | 0x00 | Reserved fields, all fixed to 0. |
|  | TX Byte[12..14] | h,t,l | Current setpoint InTempSet. |
| wl_handle_query_status / 0x2B frame | TX Byte[2] | 0x01 | Fixed valid flag. |
|  | TX Byte[3] | 0x01 | Current unit is Fahrenheit. |
|  | TX Byte[3] | 0x02 | Current unit is Celsius. |
|  | TX Byte[4..7] | 0x00 | Reserved fields, fixed to 0. |
|  | TX Byte[8..10] | h,t,l or 0x09,0x06,0x00 | Main displayed temperature. Source depends on DisplayMode. Special values report 960. |
|  | TX Byte[11..13] | h,t,l or 0x09,0x06,0x00 | probe1 temperature. Special values report 960. |
|  | TX Byte[14] | 0x00 | probe1 is a normal temperature. |
|  | TX Byte[14] | 0x02 | probe1 is special, abnormal, or disconnected. |
| wl_handle_query_status / 0x3B frame | TX Byte[2..4] | 0x09,0x06,0x00 | probe2 is unsupported; fixed report value 960. |
|  | TX Byte[5..14] | 0x00 | Reserved extension fields, fixed to 0. |
| wl_handle_query_set_all | RX Byte[1] | 0x0D | Query all set temperatures. |
|  | RX Byte[2] | 0x01 | Valid query; returns a 24-byte frame. |
|  | RX Byte[2] | Other | Returns without acknowledgement. |
|  | TX Byte[2..19] | 0x00 | Other setpoint channels are unused and fixed to 0. |
|  | TX Byte[20..22] | h,t,l | Current InTempSet as three digits. |
| wl_handle_query_act_all | RX Byte[1] | 0x0E | Query all actual temperatures. |
|  | RX Byte[2] | 0x01 | Valid query; returns a 24-byte frame. |
|  | RX Byte[2] | Other | Returns without acknowledgement. |
|  | TX Byte[2..4] | probe1 h,t,l or 0x09,0x06,0x00 | probe1 actual temperature. Special values report 960. |
|  | TX Byte[5..19] | 0x09,0x06,0x00 repeated by group | Actual temperature channels 2..6 are unsupported and fixed to 960. |
|  | TX Byte[20..22] | Main h,t,l or 0x09,0x06,0x00 | Current main displayed temperature. Special values report 960. |
| wl_dispatch / WL_CMD_BT_STATUS | RX Byte[1] | 0x24 | Bluetooth status report command. |
|  | RX Byte[2] | 0x01 | Stores s_bt_status=1 and calls UI_SetBluetoothConnectionState(1); Bluetooth is treated as connected. No reply. |
|  | RX Byte[2] | Other | Stores the value and calls UI_SetBluetoothConnectionState(0); Bluetooth is treated as disconnected. No reply. |
| wl_handle_fw_info | RX Byte[1] | 0x5F | Firmware/capability information query. |
|  | RX Byte[2] | 0x01 | Valid query; returns a 21-byte capability frame. |
|  | RX Byte[2] | Other | Returns without acknowledgement. |
|  | TX Byte[2] | 0x01 | Fixed valid flag. |
|  | TX Byte[5] | 0x01 | volume field, per code comment. |
|  | TX Byte[6] | 0x01 | type field, per code comment. |
|  | TX Byte[7] | 0x01 | probe count; indicates support for one probe. |
|  | TX Byte[10..12] | 0x01,0x07,0x05 | grill min=175. |
|  | TX Byte[13..15] | 0x04,0x05,0x05 | grill max=455. |
|  | TX Byte[16..19] | 0x00 | Reserved fields. |

3. Common Frame Examples

| Scenario | TX Frame | Effect / Reply |
| --- | --- | --- |
| Power on | FE 01 01 FF | If idle, enters start_up; replies FE 01 01 FF. |
| Power off | FE 01 02 FF | If not idle, enters shutdown; replies FE 01 02 FF. |
| Setpoint +5 | FE 03 01 05 00 00 FF | Byte[3]=5; Byte[4..5] are ignored; replies FE 03 01 h t l FF. |
| Setpoint to 175 | FE 05 01 01 07 05 FF | Writes 175 internally; echoes FE 05 01 01 07 05 FF. |
| Switch to Fahrenheit | FE 09 01 FF | system_data.units=unitF; replies FE 09 01 FF. |
| Switch to Celsius | FE 09 02 FF | system_data.units=unitC; replies FE 09 02 FF. |
| Query status | FE 0B 01 FF | Returns 0x1B, 0x2B, and 0x3B frames. |
| Bluetooth connected report | FE 24 01 FF | Sets Bluetooth connection input to connected; no reply. |

4. Integration Notes

- wireless.c has no checksum/CRC and no fixed command-length table. Most handlers only check minimum length or rx[2].

- 0x03 and 0x04 require len>=7, but only Byte[2] and Byte[3] are used. Byte[4..5] do not participate in calculation.

- 0x05 echoes raw rx[3..5]. If the input is out of range, internal InTempSet is clamped, but the reply does not show the clamped value.

- 0x09 only writes system_data.units and notifies local interaction. It does not convert the temperature array the way the key handler does.

- 0x24 has no ACK. Only rx[2]==0x01 is treated as connected; all other values are treated as disconnected.

- WirelessTask is currently not scheduled in main.c. Confirm this first during integration.